# ESP32 Quiz System Code

#include <Arduino.h>
#include <SD.h>
#include <SPI.h>
#include "Audio.h"
#include <WiFi.h>
#include <HTTPClient.h>

// Wi-Fi Credentials
const char* ssid      = "Galaxy F15 5G 838C";
const char* password  = "654292362";

// Flask Server URL
const char* serverURL = "http://192.168.84.229:5000/upload";

// SD Card Pins (FSPI)
#define SD_CS    35
#define SD_SCK   36
#define SD_MISO  38
#define SD_MOSI  37
SPIClass sdSPI(FSPI);

// Audio Output Pins
Audio audio;
#define BCLK_PIN 11
#define LRC_PIN  12
#define DIN_PIN  10

// Button Pins
#define BTN_A        14
#define BTN_B        13
#define BTN_C        47
#define BTN_D        21
#define BTN_CONFIRM  15

// Quiz Settings
const int totalQuestions = 5;
int currentQuestion = 1;

void playFile(String path);
void waitForAudio();
void waitForConfirm();
char detectOption();
void uploadAnswersToServer();

void setup() {
    Serial0.begin(115200);
    delay(1000);

    WiFi.begin(ssid, password);
    Serial0.print("Connecting to WiFi");
    while (WiFi.status() != WL_CONNECTED) {
        delay(500);
        Serial0.print(".");
    }

    Serial0.println("WiFi connected!");
    Serial0.println(WiFi.localIP());
    delay(500);

    sdSPI.begin(SD_SCK, SD_MISO, SD_MOSI, SD_CS);
    if (!SD.begin(SD_CS, sdSPI)) {
        Serial0.println("SD card failed!");
        while (1);
    }

    audio.setPinout(BCLK_PIN, LRC_PIN, DIN_PIN);
    audio.setVolume(21);

    pinMode(BTN_A, INPUT_PULLUP);
    pinMode(BTN_B, INPUT_PULLUP);
    pinMode(BTN_C, INPUT_PULLUP);
    pinMode(BTN_D, INPUT_PULLUP);
    pinMode(BTN_CONFIRM, INPUT_PULLUP);

    playFile("/welcome.mp3");
    waitForAudio();
}

void loop() {
    if (currentQuestion > totalQuestions) {
        playFile("/completed.mp3");
        waitForAudio();
        Serial0.println("Quiz Completed.");
        uploadAnswersToServer();
        while (1);
    }

    String qfile = "/ques" + String(currentQuestion) + ".mp3";
    String ofile = "/opt"  + String(currentQuestion) + ".mp3";

    playFile(qfile);  waitForAudio();
    playFile(ofile);  waitForAudio();
    playFile("/press_option.mp3");  waitForAudio();

    char selected = detectOption();
    Serial0.printf("Selected: %c\n", selected);

    String selFile = "/selected_";
    selFile += tolower(selected);
    selFile += ".mp3";

    playFile(selFile);
    waitForAudio();

    playFile("/press_confirm.mp3");
    waitForAudio();

    waitForConfirm();

    playFile("/confirmed.mp3");
    waitForAudio();

    File file = SD.open("/answers.txt", FILE_APPEND);
    if (file) {
        file.printf("Q%d: %c\n", currentQuestion, selected);
        file.close();
        Serial0.printf("Logged: Q%d: %c\n", currentQuestion, selected);
    } else {
        Serial0.println("Failed to write to answers.txt");
    }

    currentQuestion++;
}

void playFile(String path) {
    Serial0.printf("Playing: %s\n", path.c_str());
    audio.connecttoFS(SD, path.c_str());
}

void waitForAudio() {
    while (audio.isRunning()) {
        audio.loop();
    }
}

char detectOption() {
    while (1) {
        audio.loop();
        if (digitalRead(BTN_A) == LOW) return 'A';
        if (digitalRead(BTN_B) == LOW) return 'B';
        if (digitalRead(BTN_C) == LOW) return 'C';
        if (digitalRead(BTN_D) == LOW) return 'D';
    }
}

void waitForConfirm() {
    while (1) {
        audio.loop();
        if (digitalRead(BTN_CONFIRM) == LOW) {
            Serial0.println("Confirm button pressed");
            delay(300);
            return;
        }
    }
}

void uploadAnswersToServer() {
    File file = SD.open("/answers.txt");
    if (!file) {
        Serial0.println("answers.txt not found!");
        return;
    }

    String fileContent = "";
    while (file.available()) {
        fileContent += (char)file.read();
    }
    file.close();

    if (WiFi.status() == WL_CONNECTED) {
        HTTPClient http;
        http.begin(serverURL);
        http.addHeader("Content-Type", "text/plain");

        int httpResponseCode = http.POST(fileContent);

        if (httpResponseCode > 0) {
            Serial0.printf("File sent! Server response: %d\n", httpResponseCode);
            String response = http.getString();
            Serial0.println("Server: " + response);
        } else {
            Serial0.printf("HTTP error code: %d\n", httpResponseCode);
        }

        http.end();
    } else {
        Serial0.println("WiFi disconnected before upload.");
    }
}